| ## OSI Model - 7. Application Layer - What is needed to make application ‘network aware’ - 6. Presentation - Used to convert data in a format that computer can read - 5. Session - The connection between two systems - 4. Transport - Disassembles data into packets to be sent across network - Ensures data is recombined in correct manner - 3.Network - Logical Addresses - IP addresses - Routers - 2. Data link - Anything that works with a MAC address - Network Cards - Switches - 1. Physical - Cables | ## TCP/IP Model - 4. Application - Application, Presentation, Session layer of OSI Model - Anything that has to do with the application itself - Email, FTP, Telnet, SSH... - 3. Transport (& session kinda) - Assembly/Disassembly - TCP/UDP - Whatever it takes to get from one application to the next - 2. Internet - IP Addresses - Routers - 1. Network Interface (Link Layer) - Physical Cabling - Network Cards - MAC Address - Data Link and Physical of OSI Model (anything that has to do with hardware) |
| --- | --- |

## Network Interface Card (NIC) (Layer 2 Data Link)

| ### Frame - 1500bytes - Recipient MAC address - Sender’s MAC address - Type - Data - FCS - FCS - 4-bytes long - Process - NIC takes the payload - adds the FCS, - Adds Dest MAC and Source MAC - - Then sends frame through cable to a hub/switch in the network - The switch sends Unicast frames to the destination address - The switch sends Broadcast frames to every system on the network | ### MAC Address - 48-bit hexadecimal - First 6 are NIC manufacturer OUI (Organizationally Unique Identifier) - Physical address of NIC - Unique ID across all network devices ## Unicast vs. Broadcast - Unicast - Frames forwarded to specific MAC address - Broadcast - Frames forwarded to all devices in the broadcast domain - FF-FF-FF-FF-FF-FF |
| --- | --- |

## IP Addressing (Layer 3 Network Layer)

- Logical Addressing protocol for TCP/IP

- Makes sure that a piece of data gets to where it needs to go

- Unique identifier for each device on the network

### Routers

- Routers use IP addresses to forward packets where they need to go

- Connects multiple local area networks (LANs)

- Uses a routing table

| ## Transport Layer (Layer 4) - Transport protocol breaks up data into segments or datagrams - Gives each segment a sequence number - Transmission Control Protocol (TCP) - Connection Oriented - User Datagram Protocol (UDP) - Connectionless - Datagrams dont use sequence numbers |  |
| --- | --- |

Ports

- Unique to applications used on the internet

- First 1024 are ‘well known ports’

## Network Topologies

- Bus

  - Network all on one line

- Ring

- Star

  - Physical topology

- Star-Bus

  - Hybrid

  - Primary topology used today

- Mesh

  - Everyone connects to everyone

- Physical vs. Logical

## Cabling

### Coaxial

- RG-58

  - 50ohms

  - Networking

  - BNC

  - Old type

- RG-59

  - 75ohms

  - CableTV, Cable Modems Analog Video, CCTV

  - F Type

  - Short distance runs

- RG-6

  - 75ohms

  - CableTV, Cable Modems

  - F Type

  - Most common used today

  - Thicker, longer distance runs

### Twisted Pair

- Shielded

- Unshielded

- Connectors

  - RJ-45

  - RJ-11

- EIA/TIA 568A

- EIA/TIA 568B

  - Modern standard

- Solid Core vs Stranded Core

- Modern has 4 pairs

- CAT Ratings

### Fiber Optic

- Multimode

  - LED signals

  - Orange

  - Shorter distances

- Single mode

  - Lasers

  - Yellow

  - Longer distances

- Duplex

  - Two connectors

- Connectors

  - ST

    - BNC Type

    - Old - Duplex

  - SC

    - Square

    - Old - Duplex

  - FC

    - Screws in (threaded)

    - Old - Duplex

  - LC

  - MT-RJ

- Polishing

  - PC (Physical Contact)

    - Flat but rounded

  - UPC (Ultra Physical Contact)

    - Rounded

  - APC (Angled Physical Contact)

    - At an angle

  - Use UPC or APC for better quality

### Fire Rating

- Plenum Rating

  - Highest rating

- Riser

  - Between floors

- PVC - (non plenum rated)

  - No rating

### Legacy Network Connections

- Serial

  - RS-232

  - DB-Type

    - DB-25

    - DB-9

- Parallel ports

  - IEEE 1284

### IEEE

- Defines industry-wide standards that promote the use and implementation of networking technology

## Ethernet 802.3

- Nomenclature

- (10Base5, 1000Base…)

- Speed in mb/s

- Base/Broad

  - Base = 1 Channel

  - Broad = multichannels

- Length of cable

  - 10BaseT is most common

### Ethernet Frames

- 1500Bytes long

- Preamble

  - Alternating 1’s and 0’s

- Destination MAC

- Source MAC

- Data/Ether Type

  - What kind of data

- DATA

  - 64 byte/octet minimum

    - Pad Fills up the rest of the data

  - 1522 bytes/octets maximum

  - Jumbo Frame

    - 9000bytes

    - Very high speed networks

  - MTU (Maximum Transmission Unit)

- FCS (Frame Check Sequence)

  - 32 bit

  - Used for Error Protection

| ### Ethernet Basics ### 10Base5 - Drop cable - Uses BNC Connector? - Requires Terminating Resistor - Not used anymore - 10Base2 - Use BNC T-Connector - Requires Terminating Resistor - Can support up to 30 clients. - 10BaseT - Old - 10Mbps - 100M - 1024 nodes - Star-bus: physical star, logical bus - Cat 3 or better UTP cabling with RJ-45 - 10BaseFL - 10Mbps - 2000m - 1024 node limit. - Star-bus: Physical star, logical bus - Multimode fiber-optic cabling - ST or SC connectors. - CSMA/CD - Carrier Sense Multiple Access/Collision Detection - Listen on the wire - Uses random numbers to see who goes first when collision detected - Old/no longer needed thanks to full duplexing - Hubs - Essentially a repeater - Sends frames out to all devices - Switches replace hubs - Switches - Replaces hubs - Switches forward frames based on MAC address - By Making a MAC address table - Use Crossover cables (unless have auto-sensing ports) - Terminating Twisted Pair - Crossover - Have different ends - Connects switches - Suitable for direct link between two computers - Straight Through (Patch Cable) - Identical ends - Suitable for connecting workstations to network devices - 568A or 568B | ### Modern Ethernet - 100BaseT - 100BaseT4 - Cat3 - 100BaseTX - Cat5 and Cat5e - Became 100BaseT standard - 100Mbps - 100Meters - 1024 Node limit - Cat5 or better - RJ-45 - Saw the switch from hubs to switches and half-duplex to full-duplex - 100BaseFX - 100Mbps - 2km - 1024 node limit - Star-bus: Physical star, logical bus. - Multimode fiber with ST or SC connectors - Gigabit Ethernet (1000BaseT/X) - 1000BaseT (802.3ab) - 1000BaseCX - Twinax coaxial cable - 25 meter - Cat5e/6 UTP - Cat6 is most common - RJ - 45 - 100m - 1000BaseX (802.3z) - 1000BaseSX - Multimode - 850nm - LC common - 220-250m - 1000BaseLX - Single mode - 1300nm - LC and SC - 5km - 10 Gigabit Ethernet - 10GBaseT - Cat6 - 55m - Cat6a - 100m - Fiber - 10GBaseSR - Multimode - 850nm - LAN - 26-400m - 10GBaseSW - Multimode - 850nm - SONET/WAN - 26-400m - 10GBaseLR - Single mode - 1310nm - LAN - 10km - 10GBaseLW - Single mode - 1310 SONET/WAN - 10km - 10GBaseER - Single Mode - 1550 nm - LAN - 40km - 10GBaseEW - Single mode - 1550 nm - SONET/WAN - 40km - Connectors - SFP, SFP+ - Small form factor transceivers - Spanning Tree Protocol - Solves looping issues. |
| --- | --- |

## 7. TCP/IP Basics

### Converting IP Addressing and binary

  - Remember 128.

### Address Resolution Protocol (ARP)

  - Sends out a broadcast address searching for the MAC address matched to the IP

  - Used when a computer knows the IP address but needs the MAC address

  - arp -a

    - ARP cache

### Classful Addressing (subnetting)

  - Internet Assigned Numbers Authority (IANA)

    - In charge of handing out IP addresses on the internet.

    - Breaks up chunks of IP addresses and hands them to the Regional Internet Registry (RIR)

      - RIR breaks up chunks and hands them to ISP’s

  - Class A

    - 255.0.0.0

  - Class B

    - 255.255.0.0

  - Class C

    - 255.255.255.0

### Classless Addressing (Subnet Masks)

  - Subnet masks splits up the IP address into Network portion and Host portion.

  - Network ID

    - Network ID is the 1’s portion

    - Cannot change.

  - Host ID

    - Host ID is the rest 0’s

    - Cannot use 0 or 255 in the host address.

    - Host uses subnet mask to determine if the destination is local or a remote network.

      - Traffic destined for outside of the local network is sent to the default gateway router.

  - CIDR (Classless Inter-Domain Routing) Subnetting

    - Subnet masks have all 1’s on the left and all 0’s on the right

    - The more subnets you have the fewer the hosts that are available.

    - Calculating # of hosts using CIDR

      - 2^(# of 0 or host bits) - 2

### Dynamic/Static IP addressing.

#### Dynamic Host Configuration Protocol (DHCP)(BOOTP on linux)

    - Automatically assigns IP addresses from available pool.

    - Computer must be a DHCP client of the DHCP server

    - DHCP Server must be on the networks broadcast domain

      - Can use your router to create a DHCP Relay which allows a DHCP Server outside of the broadcast domain

      - And each broadcast domain must have only one DHCP Server

    - DHCP Discover

      - Performed when your computer is first booted up.

      - Sends out a Broadcast Address on the MAC address of all f’s (ff:ff:ff:ff:ff:ff)

      - DHCP server responds to this broadcast

    - DHCP Offer

      - DHCP Server receives Broadcast address of all f’s

      - Responds with a Unicast address called the DHCP Offer

    - DHCP Request

      - Client receives the DHCP Offer and accepts (rejects?) the offer

      - Sends out DHCP Request back to DHCP Server.

    - DHCP Acknowledge

      - DHCP Server stores this information and keeps traffic of clients using DHCP

  - DHCP Troubleshooting

    - Check if you have it setup correctly

      - Does it have a pool of IP Addresses

      - Is the IP Address correct etc…

    - Usually a bad server when you can connect to network devices like printers but cannot access the Internet…

    - Rogue Server

      - If you ever get an IP address other than your correct network ID...

    - If you are connected to a DHCP server and still get an APIPA address, make sure its working.

## Special IP Addresses

  - Private Networks

    - 10.x.x.x

    - 172.16.x.x - 172.31.x.x

    - 192.168.x.x

    - Used on internal networks not meant to be shared on the public internet

      - NAT Device provides a barrier (hides) from the Internet

  - Loopback

    - IPv4 - 127.x.x.x

    - IPv6 ::1

    - Used to ping yourself

    - Use with Loopback Device

  - APIPA

    - 169.254.x.x

    - Used when DHCP server is unavailable/down.

    - Can only be used locally.

## IP Scenarios

  - Duplicate IP Addresses

    - Is statistically possible.

    - Router doesnt know which computer to forward packets to.

  - Duplicate MAC Addresses

    - Happens with virtual machines

    - Cant connect to anything on the network.

  - Incorrect Gateway

    - Cant connect outside of your local network (cant access internet)

    - Man in the Middle attacks with rogue device

  - Incorrect Subnet Mask

    - Every computer on the local network (broadcast domain) must have the same Subnet Mask

    - Have two terminals ping each other.

      - When one can reach the other but the other person cannot, its a misconfigured Subnet Mask

  - Expired IP Address

    - DHCP server dies and unable to renew the DHCP Lease.

    - May also have APIPA address.

    - Check your DHCP server

## 8. Routing

### Routers

  - Filter and forward packets based on IP address.

    - (Switches filter and forward packets based on MAC address)

  - Routing Table

    - Address

    - Subnet

    - Gateway

    - Interface

    - Metric

  - Default Route

    - 0.0.0.0

    - Send to default route if no other destinations available

    - ARP’s the gateway to seek MAC address of destination

  - Gateway Routers

  - Not tied to ethernet

  - Routers only care where a packet is headed

### Ports

  - TCP packets have Destination Port and Source Port

    - Destination Port

      - Determines what type of service the packet is going to

        - i.e. 80, 443, 21, etc

    - Source Port

      - Incremental number Generated as an Ephemeral port by the computer.

      - A number well past 1024 up to 65535 depending on OS

  - 0-1023 ports are called well-known ports

### Network Address Translation/Port Address Translation (NAT/PAT)

  - PAT translates internal IP addresses to an internet address and tracks the packets

  - Static NAT (Port Forwarding)

    - Sends specific traffic to one internal IP address

    - *Someone can hack you if they know your internal IP address

  - Dynamic NAT (Pooled NAT)

    - DNAT has a limited pool of internet addresses to give to a number of internal devices

- Port Forwarding:

  - Allows external devices to have internal communication through a router

  - Port triggering will open an alternative assigned port when the initial port is contacted (e.g. FTP)

### Demilitarized Zone (DMZ)

  - A spot on your network you allow access from the public

    - Webserver etc

  - Enabling DMZ when setting up port forwarding places that device outside the protection of that router.

### Static Routing

  - Routing Tables

## TCP/IP

- Ethernet frames are used by switches and routers

- TCP (Network 3 Layer)

  - Connection-oriented

    - Uses a 3-way handshake process

    - Syn, Syn-ack, Ack

- UDP

  - Connectionless-oriented

  - Low-overhead with one-way communication.

- ICMP (Layer 2 Internet)

  - (Ping)

- IGMP (Layer 2 Internet)

  - Provides multicasting support

    - Multicast addresses start with 224

Web Servers

- HTTP:

  - Port 80

  - Microsoft IIS

  - Apache (Open Source)

  - Netstat -a if web server running

- HTTPS:

  - Port 443

- FTP

  - Listens for commands on TCP:20

  - Responds with data on TCP:21

  - SFTP

    - Secure FTP

    - Uses TLS and SSL

  - TFTP

- Email

- Sending

  - SMTP (Simple Mail Transfer Protocol)

    - TCP:25

- Receiving

- POP3

  - TCP:110

  - Older

- IMAP4

  - TCP 143

- STARTTLS: 587

- STLS

  - Current approach to running secure email protocols.

- Remote Login

  - Telnet

    - Maybe oldest program on the internet.

    - Needs a telnet client and telnet server

    - Has login but unsecure (in the clear)

  - SSH

    - Secure Shell

    - Needs an ssh client and ssh server

    - Encrypted

- NTP

  - Network Time Protocol

  - Port 123

### Network Naming

- DNS

  - Domain Name Server

  - Associates IP addresses with text domain name.

  - Troubleshooting:

### Securing TCP/IP

- Encryption

- Cleartext is any unencrypted data

- Algorithms use keys to encrypt cleartext into ciphertext.

- Symmetric Encryption

  - Uses the same key to encrypt and decrypt.

- Asymmetric Encryption

  - Uses a public key / private key to encrypt/decrypt ciphertext

  - Public keys encrypt

  - Private keys decrypt

- Cryptographic Hashes

  - Used for verifying data, not for encryption

  - Hash values are always fixed size

  - Common Hashes

    - MD5

    - SHA-1

    - SHA-2

- Identification/Authentication/Authorization

  - Authentication requires sharing of something you know, something you have or something you do

  - Federated system trust is inherited from a different trusted system.

  - Authorization

    - What can you do now that you have been authenticated

    - Access Control List

      - Mandatory Access Control (MAC)

        - Uses labels

      - Discretionary Access Control (DAC)

        - Gives creators control over permissions

      - Role-based Access Control (RBAC)

        - Uses groups

  - RADIUS

    - A RADIUS client is an intermediary agent between a RADIUS supplicant and a RADIUS server

    - A RADIUS database of authenticated users and passwords may reside outside the RADIUS server.

    - Uses UDP Ports 1812-1813

      - Or uses UDP ports 1645-1646.

    - TACACS+ uses TCP port 49

  - PPP

  - Kerberos

    - Windows proprietary service

    - Authentication authorization for wired networks

    - Key Distribution Center (KDC)

      - Authentication Service (AS)

      - Ticket-granting Service (TGS)

      - Ticket Granting Ticket

        - Timestamped by client and sent to TGS

        - TGT Issues tokens based on timestamp

    - Relies heavily on timestamps

  - EAP

    - EAP enables flexible authentication

    - Used to connect to wireless networks

    - EAP-PSK

      - Everyone uses a pre-shared key

  - Protected Extensible Authentication Protocol (PEAP)

      - Has username/password login

    - EAP-MD5

      - Uses a hash

    - EAP-TLS(TTLS)

      - Uses certificates

  - Single Sign-on

    - Windows Active Directory (Most common)

    - Used in LANs primarily

    - Security Assertion Markup Language (SAML)