COMMON PORTS

Telnet - Telecommunications Network

- TCP/23

- Login remotely

- Console Access

- In-the-clear communication (not secure)

- Not best choice for production systems

SSH - Secure Shell

- tcp/22

- Looks and acts like Telnet

- Encrypted communication link

DNS - Domain Name System

- udp/53

- Converts names to IP addresses.

SMTP - Simple Mail Transfer Protocol

- TCP/25

- Server to Server mail transfer

- Also used to send mail from a device to a mail server

SFTP - Secure File Transfer Protocol

- TCP/22

- Encrypted using SSH

FTP - File Transfer Protocol

- TCP/20 (Active mode data), TCP/21 (Control)

- Transfers files between systems

TFTP - Trivial File Transfer Protocol

- UDP/69

- Simple file transfer application

- No authentication

DHCP - Dynamic Host Control Protocol

- UDP/67, UDP/68

- Automatic configuration of IP addresses, subnet mask and other options

- Requires DHCP server

- Dynamic / pooled

  - IP addresses assigned in real-time from a pool

- Each system is given a lease and must be renewed at set intervals

DHCP Reservations

- Addresses are assigned by MAC address in the DHCP server

- Quickly manage addresses from one location

HTTP HTTPS - Hypertext Transfer Protocol (secure)

- HTTP

  - TCP/80

- HTTPS

  - TCP/443

SNMP - Simple Network Management Protocol

- UDP/161

- Gather statistics from network devices

RDP - Remote Desktop Protocol

- TCP/3389

- Share a desktop from a remote location

NTP - Network Time Protocol

- UDP/123

- Every device has its own clock (switches, routers, etc)

SIP - Session Initiation Protocol

- VOIP signaling

  - TCP/5060, TCP/5061

SMB - Server Message Block

- TCP/445

- File sharing, printer sharing

- Also called CIFS (Common Internet File System)

- Direct SMB communication over TCP without NetBIOS transport

POP / IMAP

- Receive emails from email server

  - Authenticate and transfer

- POP3 - Post Office Protocol v3

  - TCP/110

  - Basically mail transfer authority

- IMAP 4 - Internet Message Access Protocol

  - TCP/143

  - Includes management of email inbox from multiple clients

LDAP - Lightweight Director Access Protocol

- TCP/389

- Store and retrieve information in a network directory

- LDAPS (Secure)

- TCP/636

- Non-standard implementation of LDAP over SSL

H.323

- TCP/1720

- VoiP Signaling

- ITU Telecommunication H.32x protocol series

- Setup and manage VoiP sessions

# THE ETHERNET FRAME

# OSI MODEL - Open Systems Interconnection Reference Model

- Layer 7 - Application

- Layer 6 - Presentation

- Layer 5 - Session

- Layer 4 - Transport

- Layer 3 - Network

  - The ‘routing’ layer

  - Internet Protocol (IP)

  - Fragments frames to traverse different networks

- Layer 2 - Data Link

  - The basic network ‘language’

    - The foundation of communication.

  - Data Link Control (DLC) protocols

    - MAC address on Ethernet

  - The ‘switching’ layer

- Layer 1 - Physical

  - Signaling, cabling, connectors

  - Not about protocols

# MAC Address

- Ethernet Media Access Control address

  - The physical address of a network adapter

  - Unique to a device

- 48 bits / 6 bytes long

  - Displayed in hexadecimal

  - First 3 bytes is the Organizationally Unique Identifier

    - (The manufacturer)

  - Last 3 bytes is the Network Interface Controller-Specific

    - (The serial number)

# i.e. 8c:2d:aa:4b:98:a7

- Layer 2 of the OSI model